SRH-2D Version 3

SRH-2D Version 3 includes both flow and mobile-bed modules. It may be used to carry out flow-only simulation as well as a coupled flow and sediment transport modeling.

The flow-only model is simple to perform and the SRH-2D preprocessor is usually sufficient to guide one through the model setup process. Users are recommended to read the first few chapters of the User’s Manual if they do not get on-site training. The User’s Manual is included with this distribution in Document/Manual-SRH2D-v2.pdf. For time-accurate unsteady simulation, please read the document in Document/Giude_on_Unsetady_Flow_Modeling_with_SRH-2D.pdf. For flows through in-stream structures, consult Document/Final_Report_FHWA_2016.pdf.

For mobile-bed simulation, the User’s Manual is still under development as of this distribution; but a draft version is to be distributed to workshop attendants. Many users, have been able to carry out the mobile-bed modeling with SRH-2D by simply following the SRH-2D preprocessor. Users are recommended to go through the tutorial cases as well as the sample sediment modeling cases to learn.

The mobile-bed module has been under research and development for many years at Reclamation and represents current state-of-the-art. A unique total-load approach was developed in which the suspended load, bedload, and mixed-load are modeled simultaneously. The module adopts the time-accurate, unsteady formulation for mobile-bed modeling. The time-accurate solutions of the governing equations include the multi-size-class, non-equilibrium sediment transport, bed elevation changes, and sub-surface bed dynamics. The secondary flow and gravity effects on the sediment movement in streams bends are automatically taken into account. A key advantage of the adopted mobile-bed methodology is that the solution is generally applicable to all flow regimes and scenarios, and the bed evolution in time is captured even if users are interested in the final equilibrium state. The drawback, however, is that a mobile-bed modeling is usually demanding on the computing time. Smaller time step is often needed for better accuracy or for stability control. In general, the maximum number of mesh size should be kept small and only flows above a cut-off discharge in a hydrograph should be used in the mobile-bed simulation to keep the simulation time reasonable.

SRH-2D has been successfully applied to a wide range of projects at Reclamation over the years; it has also been used at many external institutions and companies. The success of the past experience leads us to release the current mobile-bed version.

1. Major Capabilities with SRH-2D Version 3

SRH-2D v3 mobile-bed module has the following general features:

- Multi-Size Sediment Representation: All granular sediments may be divided into multiple size classes (non-uniform representation) which covers the entire range of sediment sizes in the model domain.

- Non-Equilibrium Transport: Each sediment size class is transported and tracked governed by a partial differential equation (the convection-diffusion type); the sediment transport rate is usually not the same as the transport capacity and is in an non-equilibrium state.

- Variable-Load: Each sediment size class may be suspended, bedload, or mixed during transport. Users may specify the load type or SRH-2D may determine the load type automatically using an empirically developed equation.

- Multiple Subsurface Layers: Bed substrate may consist of multiple subsurface layers; each layer may have different sediment contents. Bed armoring and sorting, therefore, may be simulated.

- Sediment Transport Capacity: A number of popular sediment transport capacity equations are available to use, and they determine the ultimate fate of sediment erosion or deposition in transport.

- Secondary Flow and Gravity Effects: Sediment movement velocity does not have to be the same as the depth-averaged flow velocity. The sediment velocity may deviate from the flow due to the effects of secondary flow in bends and gravity on sloping beds.

- Cohesive Sediment: Cohesive sediment may be simulated alone or in mixture with the non-cohesive sediments. The cohesive sediments are treated as one material represented by size class number one.

2. What is included in the SRH-2D Distribution Package?

- Document  This directory contains:

  - Two User and Theory Manuals for hydraulic flow and sediment transport modeling

- SRH-2D_Package  This directory contains a number of items; they include:

  - Exec_bin directory where the SRH-2D executable and relevant libraries reside.

    - A user needs to copy “SRH-2D_Package” directory to a local directory, e.g., to C:\SRH-2D_Package, and then the model may be run using the included .bat file (need to be modified)

  - Users are recommended to use the Partial-Interface mode.

    - Now, SRH-2D has been incorporated into SMS version 13 and above.

  - The “.bat” files, srh2D_Pre.bat and srh2D.bat are enclosed that may be clicked to run SRH-2D.

    - Users need to open these two files, and edit them so that they point to the correct directory where the SRH-2D_Package\Exec_bin resides.

    - Note that 32-bit versions are phased out and not provided.

  - Users may copy the two ‘.bat’ files to any directory where a project resides and run SRH-2D programs by clicking the .bat file.

  - If you have a 64-bit PC but execution of SRH-2D 64-bit leads to complaint about “.dll” error, you may google search the error and instructions will be available on how to fix the problem. Likely, you will need to install the 64-bit version of Miscrosoft Visual C++ 2010 Redistrinbutable Package (x64). This program may be found at:  http://www.microsoft.com/en-us/download/default.aspx

- Sample_Cases  This directory contains a number of tutorial cases for training purposes. They are recommended be run first for a new user.

  - Note: Some cases might be prepared using version 3.3.0. So the SIF file may need to be modified for V3.3.1. It’s easier to run V3.3.1 preprocessor first without using the SIF file for input entry.

  - All cases were prepared to run under the Partial-Interface Mode with _SIF as the input file. They are described below:

Flow_Modeling_Cases:

  - C1_Tutorial  A simple case to test the FLOW module, and learn to use SRH-2D, as explained in the Manual, Chapter 5. Users may use both the Full-Interface mode (2DM file is sufficient) or the Partial-interface mode (both 2DM and SIF files are needed). Check your results by comparing with the RES, LN1, or PT1 files.

  - C2_Tutorial  A field scale case to learn how to use the MORPH module. Study SIF file to learn how to input extra data related to bed sediment gradation. See the Manual to learn more about inputs and outputs. Note: Only the Partial-Interface mode may be used with the MORPH module; Full-Interface mode does not support MORPH.

  - C3_Sacramento_River  A practical field case, at RM 192.5 of the Sacramento river, to learn how to use SRH-2D with XMDF output format. Both Full- and Partial-Interface modes are used for the modeling. Details of this case may be found in a conference paper titled “Modeling of Erosion and Deposition at Meandering Channels” by Lai and Greimann, ASCE World Environmental and Water Resources Congress, Honolulu, Hawaii, 2008.

  - C4_Dambreak_1D  A simple case to learn how to run unsteady flow with STH-2D. See the Unsteady Flow Modeling write-up and the 2010 FISC Training class document to learn more detail about the case.

  - InStream_Structure_Cases  A number of samples cases are provided to help users to learn on how to use the instream structures modeling. These new capabilities are described in the report, Modeling In-Stream Structures and Internal Features with SRH-2D, located within the “Document” directory. Most samples case are described in the report.

Sediment_Modeling_Cases:

  - C1_Sacramento_River  The case is the same as the Sacramento River case used for flow modeling above except that the mobile-bed, sediment transport module is turned on. The case is used as a sample on how to run the sediment transport model. Note that only Partial-Interface mode is available for sediment transport modeling.

The SMS template file is used only for the Full-Interface Mode as explained in the Manual. Due to potential bugs in some early versions of SMS, the Partial-Interface Mode is recommended if you encounter problems with the Full-Interface Mode. The Partial-Interface Mode has the benefit of (1) more input options, and (2) on-screen help-instructions on how to prepare the input data. For example, the Partial-Interface offers additional modeling options such as “Infiltration” modeling, etc.

3. Key Sediment Modeling Advices

- Computing time is very high with mobile-bed modeling due to the adoption of the time-accurate unsteady modeling formulation and a few advanced sediment modeling techniques. We emphasize more on improved accuracy. A typical mobile-bed modeling run should be limited to a mesh size of up to 15,000 cells and a total  duration of up to 100 days with a given hydrograph. Always start with a very coarse mesh for test runs which may eliminate unnecessary input errors.

  - With a multi-year flow hydrograph, it is recommended that a cut-off discharge value, say q_cut, be used so that discharges below q_cut are not simulated. Smaller discharge flows will not contribute to bed morphological changes. This way, the computing time may be reduced significantly.

- Mobile-bed sediment transport modeling may subject to stability issues despite much efforts for improvements; flow modeling is much more stable. Typically, time step is still the only way to promote stability. For typical field modeling, 1 to 5 seconds of time step are usually used. If the model run diverges with a time step less than 0.1 second, most probably, it is caused by some errors in your input parameters. For mobile-bed modeling, small time step may be needed due to accuracy requirement, not stability control. Rare cases have happened that the model diverges in the middle of the model run with the correct input setup and with a reasonably small time step. For such difficult cases, one may continue the model run using “RETSART” option through the use of _DIP.dat file by reducing the relaxation parameter, e.g., relax_h=0.3.

- SRH-2D v3 allows partition of sediments into a number of size classes. The computing time is proportional to the increase in the number of size classes. Try to limit to one to seven size classes.

- The most important input parameter that affects your erosion/deposition prediction is the sediment transport capacity equation. Other important inputs include:

  - Upstream sediment supply rate; and

  - Active layer thickness

- Occasionally, you may reduce the relaxation parameter, relax_h, through _DIP.dat file to promote stability. By default, RELAX_H=0.8 is used by SRH-2D v3. You may use RELAX_H=0.3 with _DIP.dat file for a few difficult cases. Further reduction of relaxation is not advised, and code divergence (blow-up) may indicate errors in your model setup.